Customer     : TI

Card Name	  : ISE-PLC-AOM-100

Part Number : -

Date		  : 01-03-2014

Designed At  : Tessolve Semiconductor Pvt. Ltd.

The Gerber’s are in RS274x, English 2, 5 Leading Zeros Suppressed format. The NC drill data is in English 2, 5 Trailing Zeros Suppressed format.

The zip file ISE-PLC-AOM-100_FAB_20140103.zip contains the following files.

Electrical Layers:

TII DAC R10.GTL		- Layer 1 (Top / Primary Side)

TII DAC R10.G1		- Layer 2 (Ground Plane Positive)

TII DAC R10.G2		- Layer 3 (Power Plane Positive)

TII DAC R10.GBL		- Layer 4 (Bottom / Secondary Side)

Non Electrical Layers:

TII DAC R10.GTS		- Solder Mask Top Side

TII DAC R10.GBS		- Solder Mask Bottom Side

TII DAC R10.GTO			- Silkscreen Top Side

TII DAC R10.GBO			- Silkscreen Bottom Side

TII DAC R10.GM1			- Board Outline

TII DAC R10.GM2 	- Board Dimensions

TII DAC R10.GD1		- Fabrication Drawing

Drill Layers:

TII DAC R10-Plated.TXT		 - Plated through holes drill file

TII DAC R10-NonPlated.TXT		 - Non Plated through holes drill file

Other Files:

TII DAC R10.ipc	- IPC-D-356A Netlist

Layers.pdf		- PDF Plots of all layers

Fab.pdf		- PDF of Fabrication Drawing

Readme_Fab.doc	- This file

Board Specifications:

Board Size				 : 3780Mil X 2000Mil

No. of Layers			 :	4

Board Thickness			 : 1.6mm  +/- 10%

Board Finish			 : ENIG

Min. Trace width and Spacing  : 8Mil / 7.87Mil

Min. Hole Size			 : 7.874Mil

Impedance Control		 : N/A

NOTES:

  - Vias are tented in both side

  - This is not an impedance controlled board