| CUSTOMER | TI-INDIA |
| --- | --- |
| CARD NAME | BSCM CARD |
| PART NUMBER | - |
| DATE | 19-APR-2014 |
| DESIGNED AT | Tessolve Semiconductor Pvt. Ltd. |

| GERBER FORMAT | RS274X , ENGLISH 2,5 LEADING |
| --- | --- |
| DRILL FORMAT | EXCELLON ENGLISH 2,5 TRAILING |
| PACKAGE NAME | BSCM_CARD_FAB_20140419.zip |

LIST OF FILES INSIDE PACKAGE:

ELECTRICAL LAYERS :

BSCM.GTL		- Layer 1 (Top / Primary Side)

BSCM.GBL		- Layer 2 (Bottom / Secondary Side)

NON ELECTRICAL LAYERS :

BSCM.GTS		- Solder Mask Top Side

BSCM.GBS		- Solder Mask Bottom Side

BSCM.GTO		- Silkscreen Top Side

BSCM.GBO		- Silkscreen Bottom Side

BSCM.GM1		- Board Outline

BSCM.GD1		- Fabrication Drawing

DRILL LAYERS :

BSCM-Plated.TXT		  - Plated through holes drill file

BSCM-NonPlated.TXT		 - Non Plated through holes drill file

OTHER FILES :

Fabrication Testpoint Report for BSCM_20140418_6.ipc - IPC-D-356A Netlist

Layers.pdf		- PDF Plots of all layers

Fab.pdf		- PDF of Fabrication Drawing

Readme_Fab.doc	- This file

BOARD SPECIFICATIONS :

Board Size				 : 100.0 MM  X 112.0 MM

No. of Layers			 :	2

Board Thickness			 : 1.6 MM  +/- 10%

Board Finish			 : ENIG

Min. Trace width and Spacing  : 10 mil / 6 mil

Min. Hole Size			 : 12 mil

Impedance Control		 : N/A