| CUSTOMER | TI-INDIA |
| --- | --- |
| CARD NAME | BSCM CARD |
| PART NUMBER | - |
| DATE | 19-APR-2014 |
| DESIGNED AT | Tessolve Semiconductor Pvt. Ltd. |

| GERBER FORMAT | RS274X , ENGLISH 2,5 LEADING |
| --- | --- |
| PACKAGE NAME | BSCM_CARD_ASY_20140419.zip |

LIST OF FILES INSIDE PACKAGE:

BSCM.GTP		- Solder Paste Top Side

BSCM.GBP		- Solder Paste Bottom Side

BSCM.GM5		- Assembly Drawing Top Side

BSCM.GM6		- Assembly Drawing Bottom Side

BSCM.GM1		- Board Outline

OTHER FILES:

Placement.xls 	       - Pick and Place data (Placement file)

Asy.pdf		       - PDF of Assembly Drawings (Top and Bottom side)

Readme_Asy.doc	       - This file

BOARD SPECIFICATIONS:

Board Size				 : 100.0 MM  X 112.0 MM

No. of Layers			 :	2

Board Thickness			 : 1.6 MM  +/- 10%

Board Finish			 : ENIG

Min. Trace width and Spacing  : 10 mil / 6 mil

Min. Hole Size			 : 12 mil

Impedance Control		 : N/A