Customer     :	TEXAS INSTRUMENTS

Card Name	  :	305848-5776-D-DCM03_AFE4403_MCU

Part Number :     305848-5776-D

Date		  : 	11-25-2013

Designed At  : 	Tessolve Semiconductor Pvt. Ltd.

The Gerber’s are in RS274x, English 2, 5 Leading Zeros Suppressed format. The NC drill data is in English 2, 5 Trailing Zeros Suppressed format.

The zip file 305848-5776-D-DCM03_AFE4403_MCU_FAB.zip contains the following files.

Electrical Layers:

5776L01.gbr	-	Layer 1 TOP SIDE

5776L02.gbr	-	Layer 2 GND SPLIT PLANE 1

5776L03.gbr	-	Layer 3 SIGNAL 1

5776L04.gbr	-	Layer 4 SIGNAL 2

5776L05.gbr	-	Layer 5 POWER SPLIT PLANE 1

5776L06.gbr	-	Layer 6 BOTTOM SIDE

Non Electrical Layers:

5776MT.gbr		- Solder Mask Top Side

5776MB.gbr		- Solder Mask Bottom Side

5776STW.gbr		- Silkscreen Top Side

5776SBW.gbr		- Silkscreen Bottom Side

5776SPT.gbr		- Solder Paste Top Side

5776SPB.gbr		- Solder Paste Bottom Side

5776FAB.gbr		- Fabrication Drawing

Drill Layers:

5776.dr1     	 - Plated through holes drill file

Other Files:

IPCD356.ipc 			- IPC-D-356 Netlist

5776-D_net error report.txt   - Intentional Net error report (Since board outline present in all layers, it shows some short as in the net list report)

All Layers.pdf			- PDF Plots of all layers

Readme_Fab.doc			- This file

Board Specifications:

Board Size				 : 0.609”  X 0.413”

No. of Layers			 :	6

Board Thickness			 : 0.020”  +/- 10%

Board Finish			 : GOLD

Min. Trace width and Spacing  : 0.004” / 0.004”

Min. Hole Size			 : 0.006”

Impedance Control		 : NA