**********************************************************************

PCB REF.	  : TMDSEVM6657 REV:02

JOB REF	  : EINF4567A

DATE          : 06.05.2012

**********************************************************************

FILE DETAILS

**********************************************************************

Gerber format: 2,5, Leading zeroes suppressed, Absolute, ENGLISH, RS274X

Drill format: 2,5, Trailing zeroes suppressed, Absolute, ENGLISH

GERBER FILE : TMDXEVM6657_DFM ECAD 4567_eI 18-00132-02.ZIP

(ZIP FILE CONTAINING ALL GERBER FILES AND NC DRILL DATA)

GERBER FILES:

-------------

L1.GBR		- LAYER 1 PRIMARY SIDE

L2P.GBR		- LAYER 2 GND PLANE NEGATIVE

L3S.GBR		- LAYER 3 SIGNAL LAYER POSITIVE

L4P.GBR		- LAYER 4 PWR PLANE NEGATIVE

L5S.GBR		- LAYER 5 SIGNAL LAYER POSITIVE

L6P.GBR		- LAYER 6 GND PLANE NEGATIVE

L7P.GBR		- LAYER 7 GND PLANE NEGATIVE

L8S.GBR		- LAYER 8 SIGNAL LAYER POSITIVE

L9P.GBR		- LAYER 9 PWR PLANE NEGATIVE

L10S.GBR		- LAYER 10 SIGNAL LAYER POSITIVE

L11P.GBR		- LAYER 11 GND PLANE NEGATIVE

L12.GBR		- LAYER 12 SECONDARY SIDE

SMPR.GBR		- SOLDER MASK PRIMARY

SMSE.GBR		- SOLDER MASK SECONDARY

SLPR.GBR		- SILK SCREEN PRIMARY

SLSE.GBR		- SILK SCREEN SECONDARY

DRILL FILE:

-----------

PTH.DRL  		- NC DRILL DATA PTH

PTH_SLOTS.ROU	- MILL DATA PTH SLOTS

NPTH.DRL		- NC DRILL DATA NPTH

MANUFACTURING DETAILS OF  TMDSEVM6657 REV:02

1) NO. OF LAYERS           : 12L

2) BOARD SIZE              : 180.6 X 73.5 MM

3) FINAL BOARD THICKNESS   : AS PER FAB DRAWING

4) MINIMUM DRILL SIZE      : 8 MIL

5) MAXIMUM DRILL SIZE      : 138 MIL

6) MINIMUM TRACK THICKNESS : 3.5 MIL

7) MINIMUM TRACK CLEARANCE : 4 MIL

8) COPPER THICKNESS        : AS PER FAB DRAWING

9) FINISH                  : AS PER FAB DRAWING

10) MATERIAL		   : AS PER FAB DRAWING

OTHER FILES:

------------

FAB.GBR			        - FAB DRAWING

ROUT.GBR		              - OUTLINE

IPC NETLIST.IPC   	        - IPC D 356 NETLIST

4567A_TMDSEVM6657_odb++.tgz	  - ODB++ DATA

NOTE:

1)THIS IS A CONTROLLED IMPEDANCE BOARD. FOR MORE DEAILS PLEASE REFER FAB DRAWING.

2)MASK PADS HAS BEEN INCREASED BY 3MIL WITH RESPECT TO DATABASE AT CAM LEVEL AS PER REQUEST

3)MASK PADS HAS BEEN INCREASED IN THE BGA AREA 2MIL WITH RESPECT TO DATABASE AT CAM LEVEL AS PER REQUEST

4) Cut-outs present for details refer fab drawing.

5) Einfochips logo has been Changed at Gerber level for more  clarity.

6) PTH Slots present in this board.