| CUSTOMER | TI |
| --- | --- |
| CARD NAME | TIDA-00702-REVB |
| PART NUMBER | TS11894BC0 |
| DATE | 12/03/15 |
| DESIGNED AT | Tessolve Semiconductor Pvt. Ltd. |

| GERBER FORMAT | RS274X , ENGLISH 2,5 LEADING |
| --- | --- |
| DRILL FORMAT | EXCELLON ENGLISH 2,5 TRAILING |
| PACKAGE NAME | TS11894BC0_TIDA-00702-REVB_FAB_20151203.zip |

LIST OF FILES INSIDE PACKAGE:

ELECTRICAL LAYERS :

TIDA-00702-RevB.GTL		- Layer 1 (Top / Primary Side)

TIDA-00702-RevB.GBL		- Layer 2 (Bottom / Secondary Side)

NON ELECTRICAL LAYERS :

TIDA-00702-RevB.GTS		- Solder Mask Top Side

TIDA-00702-RevB.GBS	- Solder Mask Bottom Side

TIDA-00702-RevB.GTO	- Silkscreen Top Side

TIDA-00702-RevB.GBO	- Silkscreen Bottom Side

TIDA-00702-RevB.GM1	- Board Outline

TIDA-00702-RevB.GD1	- Fabrication Drawing

DRILL LAYERS :

TIDA-00702-RevB-RoundHoles-Plated.TXT - Plated through holes drill file

TIDA-00702-RevB-SlotHoles-NonPlated.TXT - Non Plated slot Route file

OTHER FILES :

Fabrication Testpoint Report for TIDA-00702-RevB.ipc - IPC-D-356 Netlist

Layers.pdf		- PDF Plots of all layers

Fab.pdf		- PDF of Fabrication Drawing

Readme_Fab.doc	- This file

BOARD SPECIFICATIONS :

Board Size				 : 96.25MM  X 82MM

No. of Layers			 :	2

Board Thickness			 : 1.6MM  +/- 10%

Board Finish			 : ENIG

Min. Trace width and Spacing  : 10Mil / 7.995Mil

Min. Hole Size			 : 10Mil

Impedance Control		 : N/A